# CSA07 — Computer Networks

# Assessment Tool 2 — Case Study

Routing Protocol Failures — NimbusEdge Technologies Network Outage

Student Name: SHANUKA S K     Reg. No.: 1911260108

# Q1. RIP v2 — Chennai HQ Campus

## a) Understanding of the Case

Problem statement: A newly provisioned Delivery-department subnet at Chennai HQ became completely unreachable, and no monitoring alert fired — the fault surfaced only through client support tickets.

What happened, reconstructed from the Change Log and Incident Report:

- CHG-0221 (5 Feb 2024): a temporary chain of VLAN gateways was added at Chennai HQ to support a client-onboarding project for the Delivery department.

- CHG-0224 (9 Feb 2024): the new Delivery subnet was provisioned behind this chain of gateways.

- Each VLAN gateway in the chain is itself a routed hop under RIP v2. Stacking enough of them behind the core switch pushed the Delivery subnet to 16 hops from the core — one hop past RIP's usable limit.

- Because RIP does not generate an error for an unreachable route, it simply stopped advertising the Delivery subnet. No link went down, no interface flapped, and no alarm was raised — the network 'looked' healthy to standard monitoring.

- Incident Report timeline: 09:42 IST — first client ticket (Delivery team cannot reach the new subnet); 10:15 IST — NOC engineers confirm the subnet is simply absent from the HQ core switch's routing table.

- Emergency fix: CHG-0256 (18 Mar 2024) — a manual static route was inserted as a stop-gap during the outage.

The key issues, therefore, are: (1) the causal change (the VLAN gateway chain), (2) the technical trigger (hop count crossing 15), and (3) the detection gap (RIP's silent-drop behaviour combined with monitoring that only watched physical link state, not routing-table content).

## b) Application of Concepts

RIP v2 is a distance-vector protocol whose only metric is hop count, capped at 15 usable hops. Its behaviour step by step, applied to this case:

- 1. Each router that re-advertises a route to a neighbour increments the hop count by 1 before sending the advertisement.

- 2. The VLAN gateway chain inserted between the HQ core switch and the new Delivery subnet added extra routed hops that did not exist for the older Finance/HR subnets.

- 3. Once the chain of advertisements reached 16 hops, RIP applies its defined 'infinity' metric: any route with a hop count of 16 is treated as unreachable.

- 4. RIP does not send a distinct 'route unreachable' notification — it simply omits the /24 (or whatever the subnet mask) from the next periodic routing update, so the route disappears from downstream routing tables without any explicit signal.

- 5. Consequently, packets destined for the Delivery subnet had no route to follow anywhere upstream of the point where the metric hit 16, and were dropped silently at that router.

So the specific value that matters is metric = 16, RIP's definition of 'infinity' / unreachable — one more than its maximum usable hop count of 15.

## c) Analysis

No automatic alert was raised because RIP's hop-count exclusion is a normal, designed-in outcome of the protocol, not an error condition it recognises. A distance-vector protocol like RIP only ever knows 'reachable in N hops' or 'infinity' — it has no concept of a route that used to exist and no longer does, and no mechanism to flag that transition as noteworthy. This is a structural limitation of the protocol family, not a one-off human mistake in a config file: the same outcome would recur for any subnet added beyond hop 15, by any engineer, under any configuration.

This connects directly to the monitoring gap identified in the RCA (Section 4): monitoring alerted on link-down events but not on silent routing anomalies such as RIP unreachable routes or OSPF adjacency failures. In both the RIP case and the OSPF case (Q3), the physical layer stayed up while the routing layer silently failed — meaning any monitoring strategy built only around interface/link state will systematically miss this entire class of fault. The gap is architectural (what is being monitored), not operational (who was watching).

## d) Solution Design

Two complementary options:

- Redesign the addressing/subnetting scheme: instead of daisy-chaining individual VLAN gateways as sequential Layer-3 hops, consolidate the onboarding VLANs behind a single Layer-3 distribution switch (or a router-on-a-stick with sub-interfaces) that advertises the Delivery subnet in one hop from the core. This keeps every Chennai HQ subnet well inside RIP's 15-hop ceiling and is the cheaper, faster fix — it changes the campus's physical/logical hop structure, not the protocol.

- Protocol alternative: extend the multi-area OSPF migration already under way for the backbone (CHG-0233 onward) down into the Chennai HQ campus. OSPF has no hop-count ceiling, supports VLSM/route summarization, and converges by triggered LSA flooding rather than periodic updates — removing this failure mode entirely and aligning the campus with the rest of the newly redesigned network ahead of the Q3 2025 onboarding surge.

Why RIP was still a reasonable original choice: the topology-diagram notes describe the Chennai HQ campus as 'small, stable topology' — RIP v2 is simple to configure, has low router overhead, and is adequate when the number of subnets and their hop-distance from the core stay well under 15. The fault here was not that RIP was the wrong protocol for a small stable campus, but that a temporary onboarding project quietly changed the topology's depth without anyone checking it against RIP's known ceiling — which is exactly why either fix above (bounding hop depth, or moving to OSPF ahead of further growth) is justified.

## e) Clarity

Suggested structure for your final write-up (and for the diagram to attach): (1) one-line problem statement; (2) a simple hop-count diagram showing Core Switch → VLAN-GW-1 → VLAN-GW-2 → … → Delivery Subnet, with hop numbers labelled up to 16, and the point at which RIP marks it unreachable; (3) the technical reasoning from part (b); (4) your recommendation from part (d), stated as a single clear sentence.

# Q2. IGRP — Bengaluru–Pune–Hyderabad Regional Link

## a) Understanding of the Case

Problem statement: an ISP-side fault degraded the direct Bengaluru–Pune link, but IGRP kept routing traffic over the degraded path instead of switching to the healthier alternate route, causing sustained high latency for Bengaluru-based applications.

- CHG-0229 (14 Feb 2024): NOC first notifies the ISP of intermittent fiber degradation on the Bengaluru–Pune segment; the change is logged as 'Monitoring' — i.e., no route change was made at this point.

- CHG-0252 (15 Mar 2024): the ISP confirms the fault on the shared fiber segment and degradation continues intermittently, right up to the incident.

- Incident Report, 11:05 IST: Bengaluru support desk reports 600ms+ latency on applications routed via the Bengaluru–Pune link, while Hyderabad — reached via the alternate path — reports normal performance.

- The topology diagram marks the direct Bengaluru–Pune link at 2 Mbps, versus an 8 Mbps link on the Hyderabad side of the triangle — so the alternate Pune–Hyderabad–Bengaluru path had materially more headroom even before the fault, and even more so once the direct link degraded further.

- Business impact: Bengaluru-hosted client applications suffered severe latency for an extended period during the highest-impact window of the incident, until CHG-0257 (18 Mar, emergency) manually shifted traffic to the alternate path at 18:10 IST.

## b) Application of Concepts

IGRP's simplified default composite metric (K1=K3=1, K2=K4=K5=0) reduces to: Metric ≈ BW-term + Delay-term, where the bandwidth term is inversely proportional to the slowest link's bandwidth: BW-term = 10,000,000 / (bandwidth in kbps).

| Path | Bottleneck bandwidth | BW-term (10,000,000 / kbps) | Effect of fault |
| --- | --- | --- | --- |
| Direct: Bengaluru–Pune | 2,000 kbps (2 Mbps) | ≈ 5,000 | Delay term also rises as the ISP fault increases latency — composite metric worsens further |
| Alternate: Pune–Hyderabad–Bengaluru | 8,000 kbps (8 Mbps, Hyderabad-side link) | ≈ 1,250 | Extra hop adds some delay, but bandwidth term is far smaller |

Because the bandwidth term dominates the composite metric, the healthier alternate path's much lower BW-term should have outweighed its slightly higher delay (from the extra hop) even before accounting for the direct link's fault-driven delay increase. Once the ISP-side fault reduced effective bandwidth and increased delay further on the direct path, the gap should have widened further in the alternate path's favour — IGRP's own metric formula supports switching, the formula itself was not the problem.

## c) Analysis

This incident is a convergence-speed issue, not a metric-calculation issue. The composite-metric math in part (b) shows the alternate path should have won on paper. What actually delayed the switch is IGRP's reliance on periodic updates (roughly every 90 seconds) plus hold-down timers (on the order of a few minutes) that intentionally suppress rapid route changes to avoid routing loops during transient link flaps. That stability mechanism is appropriate for genuinely transient blips, but it meant IGRP kept preferring the degraded path for an extended period even after the ISP fault was confirmed as sustained — the recalculation and update propagation lagged real-time link degradation, matching the RCA's own description ('IGRP composite metric recalculation and convergence lagged the actual link degradation').

## d) Solution Design

- Short term (tuning): reduce IGRP's update interval and hold-down/invalid timers on this link so route changes propagate faster, and/or manually weight the metric (as the emergency change CHG-0257 already did) whenever a sustained ISP-side fault is confirmed.

- Longer term (protocol migration): migrate the Bengaluru–Pune–Hyderabad regional link onto the multi-area OSPF backbone already being rolled out (Areas 1–3 are assigned to these regions per the topology diagram). OSPF is link-state, floods triggered LSAs the moment an interface's cost changes, and recomputes via Dijkstra almost immediately — replacing periodic, hold-down-gated updates with event-driven convergence.

This is justified against NimbusEdge's regional scale: the network is a small triangle of three sites, well within OSPF's efficient range, and folding it into the OSPF migration already funded for the backbone avoids running a third protocol long-term. IGRP was originally expected to offer an advantage over RIP here because its composite metric (bandwidth + delay) can distinguish between links of different quality, where RIP would treat a 2 Mbps and an 8 Mbps hop as identical. That advantage is real in principle (part b confirms IGRP's own math favoured the better path), but it failed to help operationally because IGRP's slow, periodic, hold-down-protected convergence meant the 'better' route wasn't adopted quickly enough to limit real client impact.

## e) Clarity

Recommended structure: (1) the metric-calculation table above, shown explicitly; (2) a short paragraph distinguishing calculation-correctness from convergence-speed (part c); (3) your recommendation (part d) stated as tuning now / migrate later.

# Q3. OSPF — Core Backbone Migration

## a) Understanding of the Case

- CHG-0233 (20 Feb): Phase 1 — OSPF enabled on the Chennai backbone router, Area 0 defined.

- CHG-0238 (27 Feb): Phase 2 — Bengaluru router configured for Area 1, adjacency verified.

- CHG-0241 (2 Mar): Phase 2 — Pune router configured for Area 2, adjacency verified.

- CHG-0245 (8 Mar): Phase 2 — Hyderabad router configured for Area 3 — but the change log itself flags 'incorrect Area ID entered during config,' and its status was later re-tagged 'Implemented – Error Introduced.'

Trace of the fault: OSPF neighbours only become adjacent when the Area ID configured on both ends of a shared link matches. Because Hyderabad's router-side Area ID did not match what the Chennai backbone router expected on that link, the two routers never completed adjacency formation — they exchanged no link-state advertisements with each other at all. The physical fibre stayed up throughout, so nothing in standard link/interface monitoring changed state. All Hyderabad-bound traffic across that adjacency was silently dropped from 8 Mar until it was found during regional troubleshooting at 12:30 IST on 18 Mar — a black hole that persisted undetected for over a week.

## b) Application of Concepts

OSPF requires several parameters to match exactly across a link before two routers can move from Down → Init → 2-Way → Full adjacency: Area ID, Hello/Dead interval, authentication settings, and (for broadcast/NBMA networks) subnet mask. In this case, the evidence (RCA + Change Log) points specifically to an Area ID mismatch as the violated requirement — the transcription error during CHG-0245 assigned the Hyderabad interface an Area ID inconsistent with the one expected on the Chennai-side interface for that same link. An Area ID mismatch is checked at the very first Hello exchange: mismatched Hellos are simply dropped as invalid, so the two routers never even reach the 2-Way state, regardless of whether every other parameter (Hello/Dead timers, authentication) happened to be correct.

## c) Analysis

This fault was harder to detect than a physical link failure because it is invisible to interface-level and link-state monitoring: the fibre and both physical interfaces remained 'up' the entire time. Failed OSPF adjacency only shows up in the routing/OSPF-neighbour layer (e.g., a 'show ospf neighbor' check or an end-to-end traceroute), not in the link-status layer most monitoring dashboards are built around — which is exactly the same detection blind spot identified for RIP in Q1(c), and explicitly named in RCA Section 4.

The organizational gap that let a single transcription error go live is the absence of a formal change-freeze or validation window: RCA Section 4 states plainly that 'no formal change-freeze or validation window existed for the concurrent OSPF migration and client onboarding activity.' CHG-0245 was a High-risk change, approved by the CTO Office, yet nothing in the change record shows an independent post-implementation adjacency check before the ticket was closed as 'Implemented' — the error was only discovered retroactively during a live outage, ten days later, rather than being caught by a verification step built into the change process itself.

## d) Solution Design

- Technical fix: re-verify and correct the Hyderabad router's Area ID against the documented area plan (Area 3, per the topology diagram), then confirm the neighbour reaches OSPF Full state on the Chennai-facing interface before closing the change.

- Process safeguard: require (i) a second engineer's peer review of any OSPF Area ID assignment before implementation, and (ii) a mandatory adjacency-verification checklist step (capturing OSPF neighbour state, not just interface up/down) as a closing condition on every OSPF-related change ticket — directly addressing the validation-window gap named in the RCA.

Why NimbusEdge is migrating to multi-area OSPF in the first place: unlike RIP (hop-count-limited, slow periodic updates) and IGRP (Cisco-proprietary, similarly periodic/hold-down-gated), OSPF is a link-state protocol that converges quickly via triggered LSA flooding and Dijkstra recomputation, has no hop-count ceiling, and — critically for a company adding sites ahead of a Q3 2025 client-onboarding surge — its multi-area design keeps each region's link-state database and flooding domain contained within its own area, giving hierarchical scalability that neither RIP nor IGRP can offer at this trajectory of growth.

Before / After — Hyderabad Router OSPF Configuration StateBefore / After — Hyderabad Router OSPF Configuration State

| State | Hyderabad-side Area ID | Chennai backbone-side Area ID | Adjacency result |
| --- | --- | --- | --- |
| Before (8–18 Mar) | Misconfigured (transcription error, CHG-0245) | Area 0 / Area 3 boundary as designed | No adjacency — silent black hole |
| After (corrected, CHG-0258, 18 Mar) | Area 3 (matches design) | Area 0 / Area 3 boundary as designed | Adjacency restored — confirmed 17:10 IST |

## e) Clarity

Use the before/after table above alongside a redrawn extract of the topology diagram highlighting only the Chennai–Hyderabad link and its Area 0/Area 3 boundary, to visually show where the mismatch broke adjacency.

# Q4. BGP — ISP Peering and Failover

## a) Understanding of the Case

- CHG-0249 (11 Mar 2024): LOCAL_PREF values for ISP-A/ISP-B outbound preference were reviewed and modified — but the log explicitly notes this was 'not re-tested end-to-end.' This is the last touch to the failover policy before the incident.

- 14:50 IST, 18 Mar: ISP-A (primary) reports a fiber cut on NimbusEdge's leased line. Automatic failover to ISP-B (secondary) does not occur — all outbound client-facing traffic stalls.

- 15:40 IST: a senior network architect manually intervenes to force traffic onto ISP-B.

- 16:20 IST: the ISP-B path is confirmed active and partial service restoration begins.

- Per the Client SLA Breach Notice: the overall incident window was 09:42–18:55 IST (≈9h13m downtime, site-dependent), the contracted SLA of 99.9% monthly availability was breached, and the notice explicitly names the root-cause category as a routing fault spanning internal (RIP/IGRP/OSPF) and external (BGP/ISP) segments — meaning the untested BGP failover policy is formally part of what clients were told caused their breach.

## b) Application of Concepts

BGP selects a best path using, among other attributes: LOCAL_PREF (a locally-set preference value — the higher value wins, and it is how NimbusEdge was meant to prefer ISP-A under normal conditions), AS-PATH (shorter AS-PATH is generally preferred; artificially lengthening it via prepending is a common way to make a route look less attractive as a backup), and MED (a hint exchanged with a neighbouring AS about a preferred entry point, less relevant to a simple dual-homed setup like this one).

Given the RCA's description — 'primary-path preference persisted after link failure' — the likely fault is that the LOCAL_PREF/AS-PATH policy kept ISP-A's route looking preferred in NimbusEdge's BGP table even after the physical session should have gone down, rather than that ISP-B was configured with the wrong (too-low) LOCAL_PREF outright. In other words, the policy correctly encoded 'prefer ISP-A,' but nothing tied that preference to fast detection of ISP-A's failure, so BGP kept selecting the now-broken ISP-A path until someone intervened by hand.

## c) Analysis

An untested failover policy is a higher risk in BGP specifically than in an interior gateway protocol because BGP operates across an administrative boundary — testing a real failover means coordinating (or at least tolerating the effect of) an actual outage on a link shared with an external ISP, which teams are naturally reluctant to do outside a planned maintenance window. Interior protocols like OSPF can be lab-tested end-to-end entirely within NimbusEdge's own network; a live BGP failover cannot be rehearsed the same way without real external risk, which is precisely why the policy sat unmodified and unvalidated since October 2023 and was only partially touched (CHG-0249) without a live test just a week before the incident.

Business consequence chain: an untested LOCAL_PREF/AS-PATH policy → automatic failover fails to trigger on the ISP-A fiber cut → roughly 50 minutes of stalled outbound traffic before manual intervention (14:50–15:40) and a further wait for confirmed restoration (16:20) → this single gap, compounding with the RIP, IGRP, and OSPF faults running in parallel, extends the total incident to ~9h13m → the extended downtime breaches the 99.9% monthly-availability SLA for six enterprise clients → NimbusEdge must issue a formal SLA Breach Notice, apply service credits, and absorb the reputational cost of a written admission that the outage was routing-infrastructure-caused.

## d) Solution Design

Corrected policy: keep LOCAL_PREF set to prefer ISP-A under normal conditions, but tie the loss of that preference to fast, reliable failure detection — e.g., pairing the BGP session with BFD (Bidirectional Forwarding Detection) so a fiber cut tears down the session within milliseconds rather than waiting on BGP's own (much longer) hold timers, causing the ISP-A route to be withdrawn from the table immediately. ISP-B should be pre-advertised at all times with a deliberately lower LOCAL_PREF (or a longer AS-PATH via prepending) so that once the ISP-A route is withdrawn, ISP-B is automatically the best remaining path — no manual step required.

Before / After — Outbound Path-Preference PolicyBefore / After — Outbound Path-Preference Policy

| State | ISP-A route | ISP-B route | Failure detection | Failover behaviour |
| --- | --- | --- | --- | --- |
| Before | High LOCAL_PREF, always preferred | Lower LOCAL_PREF, standby | Standard BGP hold timers only | Route not withdrawn promptly → manual intervention required |
| After (proposed) | High LOCAL_PREF, always preferred | Lower LOCAL_PREF, always advertised as standby | BFD-linked session teardown | ISP-A withdrawn within seconds → ISP-B auto-selected |

Testing cadence: adopt the RCA's own corrective action — a quarterly, scheduled-maintenance-window BGP failover test that deliberately drops the ISP-A session and confirms automatic reversion to ISP-B, with a documented pass/fail result each time, so the policy is never again allowed to sit unvalidated for months.

Why BGP — not an interior protocol — is the correct choice here: RIP, IGRP, and OSPF are all Interior Gateway Protocols, designed to exchange routes within a single administratively-controlled network. NimbusEdge's dual-ISP redundancy spans two separate, externally-owned autonomous systems, which is precisely the problem BGP (an Exterior Gateway Protocol) exists to solve — it is the only protocol in this network that supports policy-based path selection and failover across independent AS boundaries, which is why it is used exclusively at the ISP-A/ISP-B edge and nowhere else in NimbusEdge's topology.

## e) Clarity

Present the before/after policy table above alongside a short timeline (14:50 fault → attempted auto-failover → 15:40 manual override → 16:20 confirmed) to make the failover-logic gap visually obvious.